Volume A

Wireless Weather Station Technical Manual

Release 1.0

Document Control

| Item | Value |
| --- | --- |
| Document Title | Wireless Weather Station Technical Manual |
| Document ID | WWS-TM-001 |
| Revision | 1.0 |
| Status | Release |
| Author | Jim |
| Classification | Technical Reference |
| System Version | Release 1 Platform |

Revision History

| Revision | Date | Description |
| --- | --- | --- |
| 0.1 | Initial Development | Original weather station concept |
| 0.5 | Development Phase | Multi-controller architecture established |
| 0.6 | Development Phase | LoRa communications implemented |
| 0.7 | Development Phase | Solar-powered outdoor node added |
| 0.8 | Development Phase | Remote reboot and uptime monitoring added |
| 0.9 | Development Phase | Dashboard and database architecture expanded |
| 1.0 | October 2026 | Initial technical manual release |

Acronyms and Definitions

| Acronym | Definition |
| --- | --- |
| ADC | Analog-to-Digital Converter |
| BMP280 | Bosch Barometric Pressure Sensor |
| DS3231 | Real-Time Clock Module |
| EEPROM | Electrically Erasable Programmable Read-Only Memory |
| ESP32 | Espressif 32-bit Microcontroller |
| GPIO | General Purpose Input / Output |
| HTTP | Hypertext Transfer Protocol |
| I²C | Inter-Integrated Circuit |
| LCD | Liquid Crystal Display |
| LoRa | Long Range Radio |
| RTC | Real-Time Clock |
| SHT31-D | Temperature/Humidity Sensor |
| UART | Universal Asynchronous Receiver / Transmitter |
| WU | Weather Underground |

List of Figures

| Figure | Description |
| --- | --- |
| 3-1 | System Architecture Diagram |
| 4-1 | Indoor Electronics Diagram |
| 4-2 | Outdoor Electronics Diagram |
| 4-3 | Solar Power System Diagram |
| 4-4 | I²C Sensor Bus Diagram |
| 4-5 | Communications Flow Diagram |
| 4-6 | Indoor Controller Assembly |
| 4-7 | Outdoor Sensor Mast |
| 4-8 | Radiation Shield Installation |

List of Tables

| Table | Description |
| --- | --- |
| 3-1 | Controller Responsibility Matrix |
| 4-1 | Indoor Hardware Inventory |
| 4-2 | Outdoor Hardware Inventory |
| 4-3 | Arduino Mega GPIO Assignments |
| 4-4 | ESP32-S3 GPIO Assignments |
| 4-5 | ESP32-C3 GPIO Assignments |
| 4-6 | Sensor Wiring Summary |
| 4-7 | Solar Power Components |

Section 1 – Introduction

1.1 Purpose

This manual documents the design, architecture, operation, maintenance, and future development of the Wireless Weather Station system.

The system combines environmental sensing, local monitoring, wireless telemetry, Internet connectivity, database storage, and dashboard presentation into a fully integrated weather-monitoring platform.

The document is intended to serve as the primary engineering reference for:

- Construction

- Maintenance

- Troubleshooting

- Firmware modification

- Future expansion

1.2 System Description

The weather station is built around a distributed architecture based on three cooperating controllers:

Arduino Mega

The Arduino Mega functions as the primary supervisory controller.

Responsibilities include:

- User interface management

- Sensor integration

- EEPROM management

- RTC management

- Min/max tracking

- Outdoor-node supervision

- Data aggregation

ESP32-S3 Gateway

The ESP32-S3 serves as the Internet gateway and presentation platform.

Responsibilities include:

- Dashboard hosting

- Wi-Fi communications

- Database uploads

- Weather Underground uploads

- Status monitoring

ESP32-C3 Outdoor Node

The ESP32-C3 serves as the outdoor telemetry controller.

Responsibilities include:

- Wind-speed measurement

- Wind-direction processing

- Rainfall measurement

- LoRa communications

- Outdoor uptime tracking

1.3 Design Goals

The weather station was designed around several key objectives:

Reliability

The station must continue operating through temporary communications failures and Internet outages.

Serviceability

The system should be diagnosable without specialized equipment.

Modularity

Major subsystems must be replaceable independently.

Expandability

Future sensors and capabilities should be incorporated without redesigning the architecture.

Data Integrity

Weather observations and system status information must be preserved for analysis and long-term review.

1.4 Major Features

The current system provides:

- Temperature monitoring

- Relative humidity monitoring

- Dew-point calculation

- Barometric pressure measurement

- Wind-speed measurement

- Wind-gust measurement

- Wind-direction measurement

- Rainfall measurement

- Historical data storage

- Browser-based dashboard

- Weather Underground integration

- Automatic fault recovery

- Solar-powered outdoor operation

Section 2 – System Overview

2.1 System Functions

The station performs four major functions:

Plain Text

Measure

Transmit

Display

Archive

Environmental data is acquired from indoor and outdoor sensors, transmitted through internal communications links, displayed locally and remotely, and archived within a database.

2.2 Environmental Measurements

The station currently measures:

| Parameter | Source |
| --- | --- |
| Temperature | SHT31-D / BMP280 |
| Humidity | SHT31-D |
| Dew Point | Calculated |
| Pressure | BMP280 |
| Wind Speed | Anemometer |
| Wind Gust | Anemometer |
| Wind Direction | Wind Vane |
| Rainfall Rate | Rain Gauge |
| Daily Rainfall | Rain Gauge |
| Outdoor Uptime | ESP32-C3 |

2.3 Data Path

Environmental measurements traverse the following path:

Plain Text

Sensors

↓

ESP32-C3

↓

LoRa

↓

Arduino Mega

↓

UART

↓

ESP32-S3

↓

Dashboard

Database

Weather Underground

This architecture separates measurement from presentation and storage.

2.4 Operational Cycle

The normal operating cycle is:

- Outdoor node acquires measurements.

- Outdoor node transmits LoRa packet.

- Arduino Mega parses telemetry.

- Arduino Mega combines indoor and outdoor data.

- Mega generates weather packet.

- ESP32-S3 receives packet.

- Dashboard updated.

- Database updated.

- Weather Underground updated.

The cycle typically repeats every five seconds.

Section 3 – System Architecture

3.1 Architectural Overview

The weather station utilizes a three-controller distributed architecture.

This design distributes responsibilities across specialized controllers rather than placing all functions on a single device.

Advantages include:

- Improved reliability

- Lower software complexity

- Easier debugging

- Improved scalability

Figure 3-1. System Architecture Diagram

[PLACEHOLDER – INSERT SYSTEM ARCHITECTURE DIAGRAM]

Plain Text

Outdoor Sensors

↓

ESP32-C3

↓ LoRa

Arduino Mega

↓ UART

ESP32-S3

↓

Dashboard

Database

Weather Underground

3.2 Controller Responsibility Matrix

| Controller | Primary Responsibilities |
| --- | --- |
| Arduino Mega | User interface, sensing, recovery management |
| ESP32-S3 | Dashboard, Wi-Fi, uploads |
| ESP32-C3 | Outdoor measurements and telemetry |

3.3 Indoor Controller Architecture

The indoor controller subsystem consists of:

- Arduino Mega

- LCD Keypad Shield

- SHT31-D

- BMP280

- DS3231 RTC

- LoRa Module

- ESP32-S3

The Mega acts as the central authority for environmental processing and operational management.

3.4 Outdoor Controller Architecture

The outdoor subsystem contains:

- ESP32-C3

- LoRa module

- Wind vane

- Anemometer

- Rain gauge

- Solar power subsystem

This subsystem operates autonomously and transmits weather observations to the indoor station.

3.5 Fault Tolerance

The architecture contains multiple layers of fault tolerance:

Sensor Redundancy

Temperature can be obtained from multiple sources.

Communications Monitoring

LoRa communication health is continuously monitored.

Automatic Recovery

The Mega can command remote reboots of the outdoor node.

Dashboard Visibility

System health information is continuously available for diagnostics.

Section 4 – Hardware Overview

4.1 Hardware Architecture

The weather station hardware is divided into:

Plain Text

Indoor Hardware

Outdoor Hardware

Power System

Communications System

4.2 Indoor Hardware Inventory

| Component | Function |
| --- | --- |
| Arduino Mega 2560 | Main controller |
| LCD Keypad Shield | Local interface |
| SHT31-D | Temperature/Humidity |
| BMP280 | Pressure |
| DS3231 RTC | Timekeeping |
| RYLR890 | LoRa communications |
| ESP32-S3 | Gateway controller |

Figure 4-1. Indoor Electronics Diagram

[PLACEHOLDER – INSERT INDOOR ELECTRONICS DIAGRAM]

Figure 4-6. Indoor Controller Assembly

[IMAGE PLACEHOLDER]

Insert:

Plain Text

Desktop inside controller.jpg

Caption:

Indoor electronics assembly showing Arduino Mega, sensors, LoRa transceiver, and ESP32-S3 gateway.

4.3 Outdoor Hardware Inventory

| Component | Function |
| --- | --- |
| ESP32-C3 | Outdoor controller |
| Wind Vane | Direction sensing |
| Anemometer | Wind-speed sensing |
| Rain Gauge | Rainfall measurement |
| RYLR890 | LoRa communications |
| Antenna | RF communications |

Figure 4-2. Outdoor Electronics Diagram

[PLACEHOLDER – INSERT OUTDOOR ELECTRONICS DIAGRAM]

Figure 4-7. Outdoor Sensor Mast

[IMAGE PLACEHOLDER]

Insert:

Plain Text

Pole mounted wind and rain sensors.jpg

Caption:

Outdoor sensor mast showing weather sensors and supporting hardware.

4.4 Arduino Mega GPIO Assignments

| Pin | Function |
| --- | --- |
| A0 | LCD keypad input |
| D10 | LCD backlight |
| D16 | LoRa TX |
| D17 | LoRa RX |
| D18 | ESP32-S3 UART TX |
| D20 | SDA |
| D21 | SCL |

4.5 ESP32-S3 GPIO Assignments

| GPIO | Function |
| --- | --- |
| GPIO18 | UART RX |
| GPIO17 | UART TX |

4.6 ESP32-C3 GPIO Assignments

| GPIO | Function |
| --- | --- |
| GPIO10 | Anemometer |
| GPIO3 | Rain Gauge |
| GPIO0 | Wind Vane ADC |
| GPIO4 | LoRa RX |
| GPIO5 | LoRa TX |

4.7 Sensor Wiring Summary

Wind Vane

| Connection | Signal |
| --- | --- |
| Pin 2 | Ground |
| Pin 3 | 3.3V |
| Pin 5 | ADC |

Anemometer

| Connection | Signal |
| --- | --- |
| Pin 3 | GPIO10 |
| Pin 4 | Ground |

Rain Gauge

| Connection | Signal |
| --- | --- |
| Pin 3 | GPIO3 |
| Pin 4 | Ground |

Figure 4-3. Solar Power System

[PLACEHOLDER – INSERT SOLAR POWER DIAGRAM]

4.8 Solar Power Hardware

| Component | Specification |
| --- | --- |
| Solar Panel | Newpowa NPA5S-12J |
| Battery | 12V 7Ah LiFePO4 |
| Controller | SOLPERK 8A IP67 |

Figure 4-8. Radiation Shield Installation

[IMAGE PLACEHOLDER]

Insert:

Plain Text

Temp and humidity shield.jpg

Caption:

Radiation shield used for temperature and humidity measurements.

4.9 Communications Hardware

The communications subsystem consists of:

- LoRa indoor module

- LoRa outdoor module

- LoRa antennas

- UART interface

- Wi-Fi interface

These components provide all data transport throughout the system.

END OF VOLUME A

Volume B

Sections 5–8

Firmware Architecture and Communications

Section 5 – Arduino Mega Firmware Architecture

5.1 Purpose

The Arduino Mega serves as the central supervisory controller for the weather station. While the outdoor ESP32-C3 acquires weather measurements and the ESP32-S3 provides network and dashboard services, the Mega functions as the operational core of the system.

Responsibilities include:

- User-interface management

- Environmental sensor processing

- RTC management

- EEPROM management

- Min/max record tracking

- Outdoor-node supervision

- Weather-data aggregation

- Communications coordination

- Fault-recovery management

The Mega is therefore the primary authority for weather-system state and operational control.

5.2 Software Architecture

The firmware utilizes a cooperative multitasking model built around the Arduino loop() function.

Rather than relying on a real-time operating system, the firmware schedules periodic activities through millisecond timing intervals using:

C++

millis()

This approach allows multiple independent activities to operate concurrently without blocking user interaction.

Major tasks include:

Plain Text

Sensor Acquisition

LCD Updates

Button Monitoring

LoRa Reception

ESP32 Packet Generation

EEPROM Management

Min/Max Tracking

Recovery Management

Diagnostics

5.3 User Interface Architecture

LCD Hardware

The user interface utilizes a standard LCD Keypad Shield mounted directly on the Arduino Mega.

The shield provides:

- 16x2 LCD display

- Five navigation buttons

- Backlight control

Buttons are connected through a resistor ladder network monitored on analog input A0.

Button Functions

| Button | Function |
| --- | --- |
| RIGHT | Navigate Forward |
| LEFT | Navigate Back |
| UP | Value Increase / Menu Navigation |
| DOWN | Value Decrease / Menu Navigation |
| SELECT | Accept / Enter |

Menu Structure

The firmware implements a hierarchical menu system.

Primary screens:

Plain Text

Super Screen

Wind Screen

Rain Screen

Clock Screen

Pressure Screen

Min Temperature

Max Temperature

Humidity Records

Pressure Records

Settings

Diagnostics

Button Test

The menu system provides complete access to operational and diagnostic functions without requiring network connectivity.

5.4 Sensor Acquisition Engine

SHT31-D

Primary measurements:

- Temperature

- Relative Humidity

The SHT31-D serves as the preferred environmental sensor because it provides both measurements from a single source.

BMP280

Measurements:

- Atmospheric Pressure

- Backup Temperature

The BMP280 primarily supplies pressure measurements but also provides secondary temperature data.

Sensor Redundancy

Temperature processing includes source-selection logic.

Priority order:

Plain Text

SHT31-D

↓

BMP280

If the primary source becomes invalid, the firmware automatically transitions to the backup source.

This improves system resilience and reduces data loss.

5.5 Min/Max Tracking Engine

The weather station continuously tracks environmental extremes.

Tracked values:

Plain Text

Minimum Temperature

Maximum Temperature

Minimum Humidity

Maximum Humidity

Minimum Pressure

Maximum Pressure

Associated timestamps are stored for every record.

Timestamp Components

Records contain:

Plain Text

Month

Day

Hour

Minute

allowing operators to determine when the extreme occurred.

5.6 Temperature Suppression System

Objective

The suppression system compensates for known afternoon temperature influences that can elevate readings beyond representative ambient conditions.

Operating Window

The correction operates during:

Plain Text

15:00

to

20:00

Correction Model

The adjustment follows a smooth sinusoidal curve.

Conceptually:

Plain Text

0

↑

Gradual Increase

↑

Maximum Reduction

↓

Gradual Decrease

↓

0

This avoids abrupt transitions while maintaining consistent environmental reporting.

5.7 EEPROM Manager

The firmware stores persistent data in EEPROM.

Stored information includes:

Plain Text

Min Temperature

Max Temperature

Min Humidity

Max Humidity

Min Pressure

Max Pressure

Altitude

Reset Information

EEPROM preservation ensures critical information survives power interruptions.

5.8 RTC Management

The DS3231 RTC provides:

- Date

- Time

- Daily rollover detection

The RTC acts as the authoritative time source for:

Plain Text

Weather Records

Database Uploads

Min/Max Timestamps

Daily Resets

5.9 Daily Reset Processing

At day rollover:

Plain Text

Store Previous Records

↓

Update Reset Timestamp

↓

Clear Daily Extremes

↓

Begin New Collection Period

This process ensures clean separation between daily observation periods.

5.10 Outdoor Communications Supervision

The Mega continuously evaluates outdoor telemetry health.

Monitoring criteria include:

Plain Text

Packet Arrival

Packet Freshness

Outdoor Uptime

Recovery State

Warning Threshold

Plain Text

30 Seconds

without valid updates.

Recovery Threshold

Plain Text

60 Seconds

without valid updates.

5.11 Remote Reboot Manager

If communications are lost, the Mega can initiate recovery.

Process:

Plain Text

Detect Timeout

↓

Request Status

↓

Issue REBOOT Command

↓

Await Acknowledgement

↓

Verify Uptime Reset

Maximum retries:

Plain Text

3

5.12 Weather Packet Generation

Once all measurements are consolidated, the Mega generates a weather-report packet for the ESP32-S3.

Typical contents include:

Plain Text

Temperature

Humidity

Dew Point

Pressure

Wind Speed

Wind Gust

Wind Direction

Rainfall

Rain Rate

Daily Rain

Outdoor Uptime

Date

Time

Transmission interval:

Plain Text

Approximately 5 seconds

Section 6 – ESP32-S3 Gateway Firmware

6.1 Purpose

The ESP32-S3 acts as the Internet gateway and data-presentation platform.

Responsibilities include:

- Wi-Fi connectivity

- Dashboard generation

- Database uploads

- Weather Underground uploads

- Status monitoring

- Communications supervision

6.2 Gateway Data Flow

Plain Text

Arduino Mega

↓

UART

↓

ESP32-S3

↓

Dashboard

Database

Weather Underground

The gateway does not generate weather data.

Instead, it receives processed information from the Mega.

6.3 UART Processing Engine

The ESP32-S3 continuously monitors UART traffic from the Mega.

The parser extracts:

Plain Text

Temperature

Humidity

Pressure

Wind Data

Rain Data

System Status

These values become dashboard variables.

6.4 Dashboard Engine

The gateway hosts a browser-accessible dashboard.

Displayed information includes:

Plain Text

Temperature

Humidity

Dew Point

Pressure

Wind

Rain

Outdoor Uptime

Weather Underground Status

The dashboard serves as the primary remote-monitoring interface.

Figure 6-1. Dashboard Data Flow

[PLACEHOLDER – INSERT DASHBOARD DATA FLOW DIAGRAM]

6.5 Weather Underground Integration

The ESP32-S3 performs all Weather Underground communications.

Functions:

Plain Text

Generate Upload Request

Transmit Data

Evaluate Success

Update Status

Dashboard classifications:

Plain Text

ONLINE

DEGRADED

OFFLINE

6.6 Database Upload Engine

The gateway uploads weather information to:

Plain Text

weatherdb.php

Data categories include:

- Environmental values

- Health metrics

- Upload status

- Time information

6.7 Moisture Analysis

The gateway calculates:

Plain Text

Dew Potential

Moisture Classification

These calculated values simplify interpretation of humidity data.

6.8 Health Monitoring

The gateway tracks:

Plain Text

Wi-Fi Status

Database Status

Weather Underground Status

Upload Success

These values provide early warning of communications issues.

6.9 Recovery Logic

If communications failures exceed acceptable limits, the ESP32-S3 may initiate self-recovery procedures.

Goals:

- Restore uploads

- Restore dashboard operation

- Restore connectivity

Section 7 – ESP32-C3 Outdoor Node Firmware

7.1 Purpose

The outdoor ESP32-C3 functions as the environmental telemetry controller.

Responsibilities include:

- Wind-speed measurement

- Wind-direction processing

- Rainfall measurement

- LoRa communications

- Uptime reporting

7.2 Sensor Interfaces

Wind Vane

Plain Text

GPIO0

ADC-based sensing.

Anemometer

Plain Text

GPIO10

Interrupt-driven pulse counting.

Rain Gauge

Plain Text

GPIO3

Interrupt-driven pulse counting.

7.3 Wind Measurement System

The anemometer generates reed-switch transitions.

Process:

Plain Text

Pulse Detection

↓

Count Accumulation

↓

Wind-Speed Calculation

7.4 Gust Processing

The firmware continuously tracks peak instantaneous wind activity.

Result:

Plain Text

Maximum Gust

reported within outgoing telemetry packets.

7.5 Wind-Direction Processing

The wind vane produces a variable resistance network.

The firmware converts:

Plain Text

ADC Reading

↓

Compass Direction

↓

Degrees

before transmission.

7.6 Rainfall Processing

The tipping-bucket rain gauge generates pulse events.

Measurements include:

Plain Text

Rain Tips

Rain Rate

Daily Rainfall

7.7 LoRa Packet Construction

The outdoor node builds telemetry packets approximately every five seconds.

Packet contents:

Plain Text

Wind Speed

Wind Gust

Wind Direction

Rain Rate

Daily Rain

Rain Tips

Uptime

7.8 Uptime Monitoring

The ESP32-C3 continuously reports uptime.

Benefits:

- Health monitoring

- Recovery verification

- Communications diagnostics

7.9 Remote Command Processing

Supported commands:

Plain Text

STATUS

REBOOT

These commands allow remote supervision by the Arduino Mega.

Section 8 – Communications Protocols

8.1 Purpose

The communications subsystem links all weather-station components into a unified architecture.

Figure 8-1. Communications Architecture

[PLACEHOLDER – INSERT COMMUNICATIONS FLOW DIAGRAM]

8.2 Communications Layers

Layer 1

Plain Text

LoRa

Outdoor telemetry.

Layer 2

Plain Text

UART

Mega-to-gateway communications.

Layer 3

Plain Text

HTTP

Internet communications.

8.3 LoRa Weather Packet

Packet identifier:

Plain Text

<OUT>

Structure:

Plain Text

WindSpeed,

WindGust,

Direction,

RainRate,

DailyRain,

RainTips,

Uptime

Update interval:

Plain Text

~5 Seconds

8.4 Status Protocol

Query:

Plain Text

<STATUS?>

Response:

Plain Text

<STATUS>

Purpose:

Verify node health when telemetry becomes stale.

8.5 Remote Reboot Protocol

Command:

Plain Text

<REBOOT>

Acknowledgement:

Plain Text

<REBOOTING>

Verification:

Plain Text

Uptime Reset

8.6 UART Weather Packet

The Mega forwards consolidated weather information to the ESP32-S3.

Fields include:

Plain Text

Temperature

Humidity

Pressure

Wind

Rain

Uptime

Date

Time

8.7 HTTP Uploads

Destination services:

Plain Text

Database

Weather Underground

The ESP32-S3 performs all upload operations.

8.8 Communications Timing

| Activity | Interval |
| --- | --- |
| LoRa Telemetry | ~5 sec |
| UART Update | ~5 sec |
| Status Check | ~30 sec |
| Reboot Trigger | ~60 sec |
| Retry Count | 3 |

8.9 Communications Health Monitoring

The architecture continuously monitors:

Plain Text

LoRa Health

Gateway Status

Upload Status

Database Connectivity

Weather Underground Connectivity

This layered monitoring structure provides robust fault detection and recovery.

END OF VOLUME B

Volume C

Sections 9–14

Database, Dashboard, Algorithms, Installation, Troubleshooting, and Maintenance

Section 9 – Database Design

9.1 Purpose

The database subsystem provides long-term storage of weather and operational data generated by the weather station. While the Arduino Mega and ESP32-S3 maintain current operating values, the database preserves historical observations for reporting, troubleshooting, trend analysis, and future dashboard enhancements.

The database serves as the permanent historical record of station operation.

9.2 Database Architecture

The database resides downstream of the weather-acquisition chain.

Plain Text

Outdoor Sensors

↓

ESP32-C3

↓

LoRa

↓

Arduino Mega

↓

UART

↓

ESP32-S3

↓

weatherdb.php

↓

Database

This separation allows weather acquisition to continue even when database services are unavailable.

9.3 Design Objectives

The database design was developed to meet several goals:

- Provide historical weather storage

- Preserve raw and corrected values

- Support future reporting tools

- Support dashboard displays

- Simplify troubleshooting

- Record system-health information

9.4 Observation Records

Each database record represents a single observation event.

Typical categories include:

Plain Text

Environmental Conditions

System Health

Upload Status

Timestamp Information

Each observation is timestamped and archived for later retrieval.

9.5 Environmental Data

Stored environmental information includes:

Temperature

Plain Text

temp

rawtemp

adjfactor

Humidity

Plain Text

humidity

dewpt

moist

dewpot

Pressure

Plain Text

presshpa

pressinhg

Wind

Plain Text

winddir

windspd

windgust

windlast

Rain

Plain Text

rainrate

rainday

raintips

9.6 System-Health Data

The database stores operational status information.

Examples:

Plain Text

Outdoor Uptime

Weather Underground Status

Last Connection

Next Upload Information

These values greatly simplify fault diagnosis.

9.7 Observation Lifecycle

Plain Text

Measurement

↓

Processing

↓

Packet Generation

↓

ESP32-S3 Upload

↓

weatherdb.php

↓

Database Insert

↓

Historical Record

9.8 Database Failure Tolerance

One design objective was fault isolation.

If database services fail:

Plain Text

Weather Collection Continues

LCD Continues

LoRa Continues

Dashboard Continues

Only historical record insertion is affected.

9.9 Future Database Applications

The current schema supports:

- Historical weather charts

- Rainfall reports

- Uptime analytics

- Reliability analysis

- Trend forecasting

without requiring major structural changes.

Section 10 – Dashboard Design

10.1 Purpose

The dashboard provides remote visibility into weather conditions and station health.

Unlike the LCD display, which is intended for local operation, the dashboard is accessible through a standard web browser and provides a complete system overview.

10.2 Dashboard Architecture

Plain Text

Arduino Mega

↓

ESP32-S3

↓

Dashboard Variables

↓

Web Interface

The dashboard reflects the most recent weather information received by the gateway.

10.3 Design Goals

The dashboard was designed to provide:

- Rapid readability

- Mobile compatibility

- System-health visibility

- Weather Underground visibility

- Near real-time updates

10.4 Main Dashboard Components

The dashboard displays:

Plain Text

Temperature

Humidity

Dew Point

Pressure

Wind

Rainfall

Outdoor Uptime

Weather Underground Status

Date/Time

Figure 10-1. Dashboard Screenshot

[IMAGE PLACEHOLDER]

Insert:

Plain Text

Dashboard ESP32 Screenshot.jpg

Caption:

Primary dashboard interface showing current weather and system status.

10.5 Temperature Display

The temperature section shows:

- Corrected temperature

- Sensor-derived values

- Suppression-adjusted values

The displayed value represents the station's preferred weather temperature.

10.6 Humidity and Dew Point

The humidity section displays:

Plain Text

Relative Humidity

Dew Point

Moisture Classification

providing a quick understanding of atmospheric conditions.

10.7 Pressure Display

Pressure information is displayed using corrected sea-level pressure.

Available units include:

Plain Text

hPa

inHg

10.8 Wind Display

Wind information includes:

Plain Text

Current Speed

Wind Gust

Direction

These values originate from the outdoor node.

10.9 Rainfall Display

The rainfall section provides:

Plain Text

Rain Rate

Daily Rainfall

Rain-Gauge Status

This allows rapid assessment of precipitation conditions.

10.10 Outdoor Node Status

The dashboard displays:

Plain Text

Outdoor Uptime

Communication Health

A rapidly growing uptime indicates healthy operation.

10.11 Weather Underground Status

Status indicators:

Plain Text

ONLINE

DEGRADED

OFFLINE

provide visibility into cloud-upload performance.

10.12 Future Dashboard Enhancements

Potential upgrades include:

- Historical charts

- Rainfall graphs

- Pressure trends

- Wind roses

- Battery telemetry

Section 11 – Environmental Algorithms

11.1 Purpose

Environmental algorithms convert raw sensor measurements into useful meteorological information.

These algorithms improve usability while preserving scientific value.

11.2 Processing Pipeline

Plain Text

Sensor

↓

Validation

↓

Correction

↓

Calculation

↓

Classification

↓

Display

Figure 11-1. Environmental Processing Pipeline

[PLACEHOLDER – INSERT ENVIRONMENTAL PIPELINE DIAGRAM]

11.3 Temperature Source Selection

Temperature may originate from:

Plain Text

SHT31-D

BMP280

The system automatically selects the preferred valid source.

11.4 Temperature Suppression Algorithm

The firmware contains a time-based correction system intended to minimize elevated readings during a known afternoon interval.

Operating Window

Plain Text

15:00 – 20:00

Characteristics

- Smooth transition

- Gradual increase

- Gradual decrease

- No abrupt step changes

Figure 11-2. Temperature Suppression Curve

[PLACEHOLDER – INSERT CURVE DIAGRAM]

11.5 Dew-Point Calculation

Dew point is calculated using temperature and humidity.

General process:

Plain Text

Temperature

+

Humidity

↓

Magnus Equation

↓

Dew Point

Figure 11-3. Dew Point Calculation Flow

[PLACEHOLDER – INSERT DEW POINT FLOW DIAGRAM]

11.6 Pressure Correction

Pressure readings are corrected using station altitude.

Inputs:

Plain Text

Observed Pressure

Altitude

Output:

Plain Text

Sea-Level Pressure

This produces weather values comparable to nearby reporting stations.

11.7 Wind Processing

The outdoor node performs:

Plain Text

Pulse Counting

Wind-Speed Calculation

Gust Tracking

Direction Processing

before transmission.

11.8 Rainfall Processing

Rainfall calculations are derived from tipping-bucket events.

Generated values:

Plain Text

Rain Tips

Rain Rate

Daily Rainfall

11.9 Moisture Classification

The system generates human-readable environmental classifications from weather conditions.

Examples may include:

Plain Text

Dry

Comfortable

Humid

Very Humid

Section 12 – Installation and Physical Deployment

12.1 Purpose

This section describes physical deployment of the complete weather station.

The installation process includes:

- Mechanical mounting

- Electronics installation

- Power-system installation

- Communications verification

12.2 Physical Assemblies

The station consists of:

Plain Text

Outdoor Sensor Mast

Outdoor Electronics Enclosure

Solar Power System

Indoor Controller

12.3 Site Selection

Ideal installation locations provide:

- Good wind exposure

- Open rainfall collection

- Reliable LoRa communications

- Sufficient solar exposure

Figure 12-1. Outdoor Sensor Mast

[IMAGE PLACEHOLDER]

Insert:

Plain Text

Pole mounted wind and rain sensors.jpg

12.4 Wind Sensor Placement

Wind sensors should:

- Avoid obstructions

- Remain well above nearby structures

- Maintain unobstructed airflow

12.5 Rain Gauge Placement

The rain gauge should:

- Remain level

- Avoid roof runoff

- Avoid splash contamination

- Remain free of overhead obstructions

12.6 Radiation Shield Installation

The shield should:

- Remain ventilated

- Remain clean

- Avoid direct solar loading

Figure 12-2. Radiation Shield

[IMAGE PLACEHOLDER]

Insert:

Plain Text

Temp and humidity shield.jpg

12.7 Solar Power Installation

Power path:

Plain Text

Solar Panel

↓

Charge Controller

↓

LiFePO4 Battery

↓

Outdoor Electronics

12.8 Indoor Controller Installation

The indoor station should be installed:

- Indoors

- Within LoRa range

- Near power

- Accessible for maintenance

12.9 Commissioning Tests

Verify:

Plain Text

LoRa Communication

Dashboard Operation

Database Uploads

Weather Underground Uploads

Sensor Function

RTC Accuracy

before placing the station into service.

Section 13 – Troubleshooting and Diagnostics

13.1 Purpose

The troubleshooting process is designed to rapidly isolate system failures.

The architecture intentionally exposes diagnostic information through:

- LCD displays

- Dashboard indicators

- Uptime reporting

- Communications status

13.2 Troubleshooting Strategy

Recommended sequence:

Plain Text

Power

↓

Controllers

↓

LoRa

↓

Dashboard

↓

Database

↓

Weather Underground

Figure 13-1. Troubleshooting Workflow

[PLACEHOLDER – INSERT TROUBLESHOOTING FLOWCHART]

13.3 Ext NO Condition

Indicates stale outdoor communications.

Possible causes:

- Outdoor-node outage

- LoRa failure

- Antenna issue

- Power-system failure

13.4 Wind Data Missing

Potential causes:

Plain Text

Sensor Failure

LoRa Failure

GPIO Wiring Problem

Inspection should begin with communications verification.

13.5 Rainfall Problems

Possible causes:

Plain Text

Debris

Bucket Binding

Wiring Failure

Pull-Up Resistor Failure

13.6 Incorrect Time

Potential causes:

Plain Text

RTC Configuration

RTC Battery

RTC Failure

13.7 Dashboard Not Updating

Possible causes:

Plain Text

UART Failure

Gateway Failure

Parser Failure

13.8 Weather Underground Offline

Potential causes:

Plain Text

Internet Outage

Wi-Fi Failure

DNS Failure

Credential Issue

13.9 Database Upload Failure

If dashboard updates continue but database records stop:

Plain Text

Likely Server-Side Failure

rather than weather acquisition failure.

13.10 Failure-Isolation Matrix

| Symptom | Likely Cause |
| --- | --- |
| Ext NO | Outdoor Communication Failure |
| No Wind | Sensor / LoRa Problem |
| Frozen Dashboard | Gateway Problem |
| OFFLINE Status | Internet Problem |
| Missing DB Records | Server Problem |
| Repeated Reboots | Power Problem |

Section 14 – Preventive Maintenance

14.1 Purpose

Preventive maintenance maximizes system reliability and minimizes downtime.

Focus areas include:

Plain Text

Inspection

Cleaning

Verification

Testing

Documentation

14.2 Recommended Maintenance Schedule

| Activity | Interval |
| --- | --- |
| Dashboard Review | Weekly |
| Outdoor Inspection | Monthly |
| Solar Cleaning | Monthly |
| Sensor Inspection | Quarterly |
| Battery Inspection | Semiannual |
| Full Functional Test | Annual |

14.3 Weekly Tasks

Verify:

Plain Text

Dashboard Updating

Outdoor Uptime Increasing

Weather Underground ONLINE

14.4 Monthly Tasks

Inspect:

Plain Text

Mast

Enclosure

Solar Panel

Mounting Hardware

Clean the solar panel as required.

14.5 Quarterly Tasks

Inspect:

Plain Text

Wind Vane

Anemometer

Rain Gauge

Sensor Wiring

Verify proper operation of each component.

14.6 Semiannual Tasks

Inspect:

Plain Text

Battery

Charge Controller

Power Wiring

Verify all connections remain secure.

14.7 Annual Tasks

Perform a complete functional verification.

Verify:

Plain Text

Sensors

LoRa

Dashboard

Database

Weather Underground

all remain operational.

Figure 14-1. Annual Maintenance Verification Flow

[PLACEHOLDER – INSERT MAINTENANCE FLOW DIAGRAM]

14.8 Post-Storm Inspection

After severe weather:

Inspect:

Plain Text

Sensors

Pole

Enclosure

Solar Panel

Antenna

Cabling

and verify normal operation.

14.9 Service Log

A maintenance record should include:

Plain Text

Date

Technician

Findings

Corrective Actions

Firmware Version

Long-term records simplify support and system evolution.

END OF VOLUME C

Volume D

Appendices A–H

Reference Material, Asset Inventory, Change History, and Future Planning

Appendix A – GPIO and Signal Reference

A.1 Purpose

This appendix provides a consolidated reference for all controller GPIO assignments, communication interfaces, sensor connections, and signal paths used throughout the weather station.

The GPIO reference serves as the primary hardware reference for:

- Troubleshooting

- Firmware development

- Hardware expansion

- Maintenance

- System replication

A.2 Arduino Mega GPIO Assignments

User Interface

| Pin | Function |
| --- | --- |
| A0 | LCD button ladder input |
| D10 | LCD backlight control |

LoRa Communications

| Pin | Function |
| --- | --- |
| D16 | LoRa TX |
| D17 | LoRa RX |

ESP32-S3 Communications

| Pin | Function |
| --- | --- |
| D18 | UART1 TX |
| D19 | UART1 RX |

I²C Bus

| Pin | Function |
| --- | --- |
| D20 | SDA |
| D21 | SCL |

A.3 ESP32-S3 GPIO Assignments

| GPIO | Function |
| --- | --- |
| GPIO18 | UART RX from Mega |
| GPIO17 | UART TX |

A.4 ESP32-C3 GPIO Assignments

| GPIO | Function |
| --- | --- |
| GPIO0 | Wind vane ADC |
| GPIO3 | Rain gauge |
| GPIO4 | LoRa RX |
| GPIO5 | LoRa TX |
| GPIO10 | Anemometer |

A.5 Sensor Interface Summary

Indoor Sensors

Plain Text

SHT31-D

BMP280

DS3231

Communications:

Plain Text

I²C

Outdoor Sensors

Plain Text

Wind Vane

Anemometer

Rain Gauge

Communications:

Plain Text

ADC + Interrupt Inputs

A.6 Signal Path Summary

Plain Text

Wind Vane

Anemometer

Rain Gauge

↓

ESP32-C3

↓

LoRa

↓

Arduino Mega

↓

UART

↓

ESP32-S3

Appendix B – EEPROM Memory Map

B.1 Purpose

EEPROM storage preserves critical station information across power cycles.

Stored information includes:

- Environmental records

- Settings

- Reset information

- Calibration parameters

B.2 EEPROM Allocation

| Address | Function |
| --- | --- |
| 0 | Min Temperature |
| 4 | Max Temperature |
| 8 | Min Humidity |
| 12 | Max Humidity |
| 16 | Min Pressure |
| 20 | Max Pressure |
| 24 | Altitude |
| 28–48 | Timestamp Records |
| 500 | Reset Flag |
| 520–523 | Last Reset Information |

B.3 EEPROM Organization

Plain Text

Environmental Records

↓

Timestamps

↓

Configuration

↓

Recovery Information

B.4 Startup Recovery

Upon startup:

Plain Text

Read EEPROM

↓

Restore Settings

↓

Restore Records

↓

Resume Normal Operation

Appendix C – Communications Protocol Reference

C.1 Protocol Layers

Plain Text

Layer 1 LoRa

Layer 2 UART

Layer 3 HTTP

C.2 OUT Packet

Purpose

Primary outdoor telemetry packet.

Structure

Plain Text

<OUT>

WindSpeed,

WindGust,

Direction,

RainRate,

DailyRain,

RainTips,

Uptime

</OUT>

C.3 STATUS Protocol

Request

Plain Text

<STATUS?>

Response

Plain Text

<STATUS>

C.4 REBOOT Protocol

Request

Plain Text

<REBOOT>

Response

Plain Text

<REBOOTING>

C.5 UART Weather Packet

The Mega forwards:

Plain Text

Temperature

Humidity

Dew Point

Pressure

Wind Speed

Wind Direction

Rainfall

Outdoor Uptime

Date

Time

to the ESP32-S3.

C.6 Communications Timing

| Activity | Interval |
| --- | --- |
| LoRa Telemetry | ~5 sec |
| UART Update | ~5 sec |
| Status Request | ~30 sec |
| Reboot Trigger | ~60 sec |
| Retry Count | 3 |

Appendix D – Database Field Dictionary

D.1 Temperature Fields

| Field | Description |
| --- | --- |
| temp | Corrected temperature |
| rawtemp | Raw sensor reading |
| adjfactor | Applied adjustment |

D.2 Moisture Fields

| Field | Description |
| --- | --- |
| humidity | Relative humidity |
| dewpt | Dew point |
| moist | Moisture classification |
| dewpot | Dew-potential classification |

D.3 Pressure Fields

| Field | Description |
| --- | --- |
| presshpa | Pressure in hPa |
| pressinhg | Pressure in inches Hg |

D.4 Wind Fields

| Field | Description |
| --- | --- |
| winddir | Direction |
| windspd | Wind speed |
| windgust | Peak gust |
| windlast | Previous direction value |

D.5 Rain Fields

| Field | Description |
| --- | --- |
| rainrate | Current rain intensity |
| rainday | Daily rainfall |
| raintips | Bucket-tip count |

D.6 Health Fields

| Field | Description |
| --- | --- |
| outuptime | ESP32-C3 uptime |
| nextupl | Upload timer |
| lastconn | Last connection |
| WUstat | Weather Underground status |

Appendix E – System Parts List (BOM)

E.1 Indoor Controller

| Qty | Component |
| --- | --- |
| 1 | Arduino Mega 2560 |
| 1 | ESP32-S3 |
| 1 | LCD Keypad Shield |
| 1 | SHT31-D |
| 1 | BMP280 |
| 1 | DS3231 RTC |
| 1 | RYLR890 LoRa Module |

E.2 Outdoor Node

| Qty | Component |
| --- | --- |
| 1 | ESP32-C3 |
| 1 | RYLR890 LoRa Module |
| 1 | Wind Vane |
| 1 | Anemometer |
| 1 | Tipping-Bucket Rain Gauge |

E.3 Solar Power System

| Qty | Component |
| --- | --- |
| 1 | Newpowa NPA5S-12J Solar Panel |
| 1 | SOLPERK 8A Charge Controller |
| 1 | 12V LiFePO4 Battery |

E.4 Spare Parts Inventory

Recommended stock:

Plain Text

ESP32-C3

ESP32-S3

LoRa Module

SHT31-D

BMP280

DS3231

RJ11/RJ12 Connectors

10K Resistors

Sensor Cable

Appendix F – Source File Inventory

F.1 Firmware Files

Arduino Mega

Plain Text

Min_Max_Temp_with_LoRa.ino

Purpose:

Plain Text

Main Weather Controller

ESP32-S3

Plain Text

ESP32_S3_Inside_Station.ino

Purpose:

Plain Text

Gateway / Dashboard

ESP32-C3

Plain Text

ESP32_C3_outside_station.ino

Purpose:

Plain Text

Outdoor Telemetry Node

F.2 PHP Files

weatherdb.php

Purpose:

Plain Text

Database Upload Handler

index.php

Purpose:

Plain Text

Dashboard Interface

dewpoint.php

Purpose:

Plain Text

Dew Point Calculations

F.3 Engineering References

Plain Text

New RJ11 and Wireless Connections.txt

Power Supply Info.txt

LoRa Output.txt

Appendix G – System Evolution and Change Log

G.1 Development Progression

The weather station evolved through several major phases:

Plain Text

Single Controller

↓

LoRa Communications

↓

Outdoor Controller

↓

ESP32-S3 Gateway

↓

Database Storage

↓

Web Dashboard

↓

Solar Power System

G.2 Major Enhancements

Distributed Controller Architecture

Benefits:

- Reduced software complexity

- Better fault isolation

- Improved maintainability

LoRa Communications

Benefits:

- Reduced wiring

- Improved deployment flexibility

- Better sensor placement

Remote Reboot Capability

Benefits:

- Reduced service visits

- Improved uptime

- Automated recovery

Database Archiving

Benefits:

- Historical records

- Trend analysis

- Reporting capability

G.3 Future Change Log

| Date | Revision | Description |
| --- | --- | --- |
| TBD | TBD | Future enhancement |

Appendix H – Future Enhancements Roadmap

H.1 Power Monitoring

Future capabilities:

Plain Text

Battery Voltage

Charge Current

Solar Voltage

Battery Health

H.2 Environmental Expansion

Potential additions:

Plain Text

Outdoor Humidity

UV Index

Solar Radiation

Air Quality

H.3 Dashboard Improvements

Potential features:

Plain Text

Historical Graphs

Trend Analysis

Interactive Reporting

Mobile Optimization

H.4 Communications Enhancements

Potential features:

Plain Text

Packet Statistics

Reliability Metrics

Signal-Quality Tracking

Advanced Diagnostics

H.5 Remote Administration

Future possibilities:

Plain Text

Remote Configuration

Remote Calibration

Remote Firmware Updates

H.6 Version 2.0 Vision

Potential Version 2.0 capabilities:

Plain Text

Solar Telemetry

Battery Telemetry

Historical Charting

Advanced Forecasting

Remote Updates

Additional Sensors

Enhanced Analytics

while maintaining the proven architecture:

Plain Text

Arduino Mega

ESP32-S3 Gateway

ESP32-C3 Outdoor Node

LoRa Communications

Dashboard

Database

End of Volume D

Volume E

Operational Reference, Recovery Procedures, and Long-Term Support

E.1 Purpose

This volume provides operational guidance that supplements the technical material presented throughout the manual. While the previous sections focus on system architecture and implementation details, this volume focuses on day-to-day operation, recovery planning, and long-term stewardship of the weather station.

The objective is to ensure that future maintenance personnel can successfully operate, troubleshoot, and preserve the system even if the original designer is no longer available.

E.2 System Startup Procedure

Cold Start

After a complete power loss, the system should start in the following sequence:

Plain Text

Power Applied

↓

Arduino Mega Starts

↓

RTC Restored

↓

Indoor Sensors Initialized

↓

ESP32-S3 Starts

↓

WiFi Connection Established

↓

ESP32-C3 Outdoor Node Starts

↓

LoRa Communications Established

↓

Dashboard Updates Begin

Normal startup should require no operator intervention.

Startup Verification Checklist

Verify:

- LCD active

- Date and time valid

- Ext OK displayed

- Outdoor uptime increasing

- Dashboard updating

- Weather Underground ONLINE

- Database records appearing

E.3 Normal Operations Checklist

The station is intended to operate continuously.

Normal indicators include:

Arduino Mega

Plain Text

Display Active

Current Weather Displayed

Button Navigation Functional

ESP32-C3

Plain Text

Outdoor Uptime Increasing

Wind Updates Present

Rain Updates Present

ESP32-S3

Plain Text

Dashboard Updating

Database Uploads Active

Weather Underground ONLINE

E.4 Recovery Procedures

Communications Failure Recovery

If outdoor communications stop:

- Observe Ext NO condition.

- Verify outdoor uptime.

- Allow automatic recovery.

- Observe STATUS requests.

- Observe REBOOT attempts.

- Verify uptime reset after recovery.

Dashboard Recovery

If dashboard updates stop:

- Verify Arduino Mega operation.

- Verify UART communications.

- Verify ESP32-S3 operation.

- Verify browser connectivity.

- Restart ESP32-S3 if necessary.

Internet Recovery

If uploads fail:

- Verify WiFi connectivity.

- Verify local Internet access.

- Verify Weather Underground status.

- Verify database connectivity.

- Observe automatic recovery.

E.5 Seasonal Inspection Recommendations

Spring

Perform:

- Rain-gauge cleaning

- Wiring inspection

- Sensor inspection

Particular attention should be given to winter weather damage.

Summer

Perform:

- Solar-panel cleaning

- Radiation-shield inspection

- Temperature verification

Excessive contamination can affect summer temperature measurements.

Fall

Perform:

- Rain-gauge cleaning

- Leaf removal

- Sensor inspection

This is particularly important for maintaining rainfall accuracy.

Winter

Inspect:

- Battery health

- Solar charging performance

- Enclosure sealing

Cold-weather operation should be verified before severe weather arrives.

E.6 Documentation Management

The following records should be preserved throughout the service life of the weather station:

Technical Documents

Plain Text

Technical Manual

User Manual

Protocol Specifications

Wiring Diagrams

Firmware Archives

Plain Text

Arduino Mega Firmware

ESP32-S3 Firmware

ESP32-C3 Firmware

Every released version should be archived.

Installation Records

Maintain:

Plain Text

Installation Photos

Wiring Photos

Enclosure Photos

Sensor Alignment Records

These records greatly simplify future troubleshooting.

E.7 Backup Strategy

Firmware Backups

Backup before:

- Feature additions

- Pin changes

- Protocol modifications

- Sensor additions

Recommended archive structure:

Plain Text

Firmware

├── Mega

├── ESP32-S3

└── ESP32-C3

Database Backups

Recommended schedule:

Plain Text

Weekly

Minimum schedule:

Plain Text

Monthly

Historical weather data becomes increasingly valuable as the archive grows.

Documentation Backups

Backup after every major revision.

Recommended archive:

Plain Text

Technical Manual

User Manual

Installation Notes

Photos

Schematics

E.8 Spare Equipment Strategy

Recommended storage inventory:

Plain Text

1 Arduino Mega

1 ESP32-S3

1 ESP32-C3

1 LoRa Module

10 RJ Connectors

2 Sensor Cables

1 Battery

Miscellaneous Resistors

The objective is rapid recovery from common failures.

E.9 Long-Term Support Philosophy

The weather station was designed around three priorities:

Plain Text

Reliability

Serviceability

Expandability

Future modifications should preserve these goals whenever possible.

New features should not compromise:

- Existing weather acquisition

- Existing communications

- Existing recovery mechanisms

E.10 Release Acceptance Checklist

Before publication of a new system release:

Hardware

- Sensor wiring verified

- GPIO assignments verified

- Solar system verified

- Communications verified

Firmware

- Arduino firmware verified

- ESP32-S3 firmware verified

- ESP32-C3 firmware verified

Dashboard

- Data updates verified

- Status indicators verified

- Mobile layout verified

Uploads

- Database uploads verified

- Weather Underground uploads verified

Documentation

- Technical manual updated

- BOM updated

- Protocol documentation updated

- Revision history updated

E.11 Release Statement

Wireless Weather Station Technical Manual

Document ID: WWS-TM-001

Release: 1.0

This document defines the architecture, implementation, operation, maintenance, troubleshooting, and future development path of the Wireless Weather Station platform.

The manual shall serve as the authoritative engineering reference for future maintenance, enhancement, support, and replication of the system.

End of Volume E

Volume F

Publication Assets, Figure Register, Approval Records, and Release Packaging

F.1 Purpose

This volume contains the publication support material required to transform the technical manuscript into a formal engineering document.

The material in this volume is intended for:

- Technical publication

- Revision management

- Figure management

- Release control

- Long-term document maintenance

This volume should remain part of all future revisions of the Technical Manual.

F.2 Figure Register

The following figures are referenced throughout the document.

Section 3 Figures

Figure 3-1

System Architecture Diagram

Purpose:

Illustrates the relationships between:

Plain Text

ESP32-C3 Outdoor Node

Arduino Mega

ESP32-S3 Gateway

Database

Dashboard

Weather Underground

Status:

Plain Text

Diagram Required

Section 4 Figures

Figure 4-1

Indoor Electronics Diagram

Purpose:

Documents indoor-controller wiring and subsystem relationships.

Status:

Plain Text

Diagram Required

Figure 4-2

Outdoor Electronics Diagram

Purpose:

Documents outdoor-controller wiring and sensor interfaces.

Status:

Plain Text

Diagram Required

Figure 4-3

Solar Power System

Purpose:

Documents solar charging architecture.

Status:

Plain Text

Diagram Required

Figure 4-4

I²C Sensor Bus

Purpose:

Documents communications between:

Plain Text

Arduino Mega

SHT31-D

BMP280

DS3231

Status:

Plain Text

Diagram Required

Figure 4-5

Communications Flow Diagram

Purpose:

Documents system communications paths.

Status:

Plain Text

Diagram Required

Figure 4-6

Indoor Controller Assembly

Status:

Plain Text

Photo Placeholder

Source:

Plain Text

Desktop inside controller.jpg

Figure 4-7

Outdoor Sensor Mast

Status:

Plain Text

Photo Placeholder

Source:

Plain Text

Pole mounted wind and rain sensors.jpg

Figure 4-8

Radiation Shield Installation

Status:

Plain Text

Photo Placeholder

Source:

Plain Text

Temp and humidity shield.jpg

Section 6 Figures

Figure 6-1

Dashboard Data Flow

Purpose:

Documents weather-data movement through the gateway.

Status:

Plain Text

Diagram Required

Section 8 Figures

Figure 8-1

Communications Architecture

Purpose:

Illustrates:

Plain Text

LoRa Layer

UART Layer

HTTP Layer

Status:

Plain Text

Diagram Required

Section 10 Figures

Figure 10-1

Dashboard Screenshot

Status:

Plain Text

Photo Placeholder

Source:

Plain Text

Dashboard ESP32 Screenshot.jpg

Section 11 Figures

Figure 11-1

Environmental Processing Pipeline

Status:

Plain Text

Diagram Required

Figure 11-2

Temperature Suppression Curve

Status:

Plain Text

Diagram Required

Figure 11-3

Dew Point Calculation Flow

Status:

Plain Text

Diagram Required

Section 12 Figures

Figure 12-1

Outdoor Sensor Mast Installation

Status:

Plain Text

Photo Placeholder

Source:

Plain Text

Pole mounted wind and rain sensors.jpg

Figure 12-2

Radiation Shield Installation

Status:

Plain Text

Photo Placeholder

Source:

Plain Text

Temp and humidity shield.jpg

Section 13 Figures

Figure 13-1

Troubleshooting Workflow

Status:

Plain Text

Diagram Required

Section 14 Figures

Figure 14-1

Maintenance Verification Workflow

Status:

Plain Text

Diagram Required

F.3 Table Register

Core Technical Tables

| Table | Description |
| --- | --- |
| 3-1 | Controller Responsibility Matrix |
| 4-1 | Indoor Hardware Inventory |
| 4-2 | Outdoor Hardware Inventory |
| 4-3 | Arduino Mega GPIO Assignments |
| 4-4 | ESP32-S3 GPIO Assignments |
| 4-5 | ESP32-C3 GPIO Assignments |
| 4-6 | Sensor Wiring Summary |
| 4-7 | Solar Hardware Inventory |

Maintenance Tables

| Table | Description |
| --- | --- |
| 13-1 | Failure-Isolation Matrix |
| 14-1 | Preventive Maintenance Schedule |

Appendix Tables

| Table | Description |
| --- | --- |
| A-1 | GPIO Reference |
| B-1 | EEPROM Memory Map |
| C-1 | Communications Timing |
| D-1 | Database Fields |
| E-1 | System Parts List |
| F-1 | Source File Inventory |

F.4 Publication Layout Standard

Paper Size

Plain Text

Letter

8.5 x 11 inches

Margins

Plain Text

1 inch

all sides.

Body Text

Plain Text

Calibri

11 pt

Heading Styles

Plain Text

Heading 1

Heading 2

Heading 3

using native Microsoft Word styles.

Tables

Plain Text

Table Grid

style recommended.

Figure Captions

All figures should use Microsoft Word Caption style.

Example:

Plain Text

Figure 8-1. Communications Architecture

Code Blocks

Recommended format:

Plain Text

Consolas

10 pt

F.5 Engineering Review Record

Architecture Review

Status:

Plain Text

APPROVED

Firmware Review

Status:

Plain Text

APPROVED

Communications Review

Status:

Plain Text

APPROVED

Database Review

Status:

Plain Text

APPROVED

Dashboard Review

Status:

Plain Text

APPROVED

Maintenance Documentation Review

Status:

Plain Text

APPROVED

F.6 Publication Readiness Checklist

Technical Content

- Sections 1–14 present

- Appendices A–H present

- Revision History current

- Acronyms current

Figures

- Architecture diagrams inserted

- Dashboard screenshot inserted

- Mast photo inserted

- Radiation shield photo inserted

Tables

- GPIO tables verified

- EEPROM map verified

- Database tables verified

- BOM verified

Final Document

- Table of Contents generated

- List of Figures generated

- List of Tables generated

- Page numbers added

- PDF version exported

- DOCX archived

F.7 Master Manuscript Structure

The Release 1.0 manuscript consists of:

Plain Text

Volume A

Front Matter

Sections 1-4

Volume B

Sections 5-8

Volume C

Sections 9-14

Volume D

Appendices A-H

Volume E

Operations and Long-Term Support

Volume F

Publication Assets and Release Material

Volume G

Engineering Notes and Design Rationale

F.8 Release Approval Page

Document Title

Plain Text

Wireless Weather Station Technical Manual

Document ID

Plain Text

WWS-TM-001

Revision

Plain Text

1.0

Status

Plain Text

Approved for Release

Author

Plain Text

Jim

F.9 Release Statement

This document establishes the authoritative engineering baseline for the Wireless Weather Station platform.

The manual documents:

- Architecture

- Firmware

- Communications

- Database Systems

- Dashboard Systems

- Installation

- Troubleshooting

- Maintenance

- Future Expansion

The intent of this publication is to ensure that the weather station can be maintained, expanded, repaired, and replicated without requiring knowledge external to the documentation set.

End of Volume F

Volume G

Engineering Notes and Design Rationale

The purpose of this volume is to preserve the engineering reasoning behind major design decisions. Future maintenance is significantly easier when system behavior can be traced back to the original design objectives.

G.1 Why a Three-Controller Architecture Was Selected

The weather station could have been implemented using a single microcontroller.

That approach was ultimately rejected because it would have created a controller responsible for:

- Weather measurements

- User interface

- Database communications

- Dashboard hosting

- LoRa communications

- Internet communications

By distributing responsibilities across three processors, each subsystem remains relatively independent.

Arduino Mega

Responsible for:

Plain Text

Weather Logic

LCD Interface

RTC

EEPROM

Recovery Management

ESP32-C3

Responsible for:

Plain Text

Outdoor Data Acquisition

LoRa Communications

Rain Processing

Wind Processing

ESP32-S3

Responsible for:

Plain Text

Dashboard

WiFi

Database Uploads

Weather Underground Uploads

This separation significantly simplifies maintenance and troubleshooting.

G.2 Why LoRa Was Selected

Several wireless technologies were considered.

WiFi

Advantages:

Plain Text

High Throughput

Easy Network Integration

Disadvantages:

Plain Text

Higher Power Consumption

Outdoor Coverage Challenges

Bluetooth

Advantages:

Plain Text

Low Cost

Simple Hardware

Disadvantages:

Plain Text

Limited Range

LoRa

Advantages:

Plain Text

Excellent Range

Low Power Consumption

Reliable Outdoor Operation

Simple Deployment

LoRa was selected because it provided the best combination of range and reliability for the outdoor sensor node.

G.3 Why Solar Power Was Adopted

The original concept assumed permanent wired power.

During development it became apparent that outdoor sensor placement and wiring flexibility would improve significantly if the outdoor node became self-powered.

Benefits achieved:

Plain Text

No power cable

Flexible installation

Reduced trenching

Improved placement options

G.4 Why Outdoor Uptime Was Added

Outdoor uptime was not part of the original design.

Troubleshooting revealed that determining whether the outdoor node had rebooted was often difficult.

The uptime counter was introduced to provide visibility into:

Plain Text

Unexpected Resets

Power Problems

Recovery Success

Communications Health

It became one of the most valuable diagnostic metrics in the entire system.

G.5 Why Remote Reboot Was Added

Field failures occasionally required access to the outdoor node.

Because the node could be mounted outdoors and operate on solar power, physical access is less convenient than access to indoor equipment.

The remote reboot protocol was created so that:

Plain Text

Communications Failures

Software Lockups

Temporary Faults

could often be recovered without a service visit.

G.6 Reliability Philosophy

The station was designed around a simple principle:

Plain Text

Collect Data First

Display Data Second

Upload Data Third

Weather acquisition should continue whenever possible.

Failures in:

Plain Text

Internet Connectivity

Database Services

Dashboard Services

should not prevent weather collection.

This principle strongly influenced the final architecture.

G.7 Design Principles

The weather station follows the principles below.

Simplicity

Prefer understandable solutions over sophisticated solutions.

Recoverability

Subsystems should recover automatically whenever practical.

Visibility

Failures should be obvious.

Documentation

Critical behaviors should be documented.

Expandability

Future sensors should be easy to add.

G.8 Long-Term Support Objectives

The project was designed with an anticipated service life measured in years rather than months.

Long-term goals include:

Plain Text

Minimal Maintenance

Predictable Operation

Simple Repair

Incremental Enhancement

The technical manual exists partly to support these objectives.

End of Volume G