Volume E

Operational Reference, Recovery Procedures, and Long-Term Support

E.1 Purpose

This volume provides operational guidance that supplements the technical material presented throughout the manual. While the previous sections focus on system architecture and implementation details, this volume focuses on day-to-day operation, recovery planning, and long-term stewardship of the weather station.

The objective is to ensure that future maintenance personnel can successfully operate, troubleshoot, and preserve the system even if the original designer is no longer available.

E.2 System Startup Procedure

Cold Start

After a complete power loss, the system should start in the following sequence:

Plain Text

Power Applied

↓

Arduino Mega Starts

↓

RTC Restored

↓

Indoor Sensors Initialized

↓

ESP32-S3 Starts

↓

WiFi Connection Established

↓

ESP32-C3 Outdoor Node Starts

↓

LoRa Communications Established

↓

Dashboard Updates Begin

Normal startup should require no operator intervention.

Startup Verification Checklist

Verify:

- LCD active

- Date and time valid

- Ext OK displayed

- Outdoor uptime increasing

- Dashboard updating

- Weather Underground ONLINE

- Database records appearing

E.3 Normal Operations Checklist

The station is intended to operate continuously.

Normal indicators include:

Arduino Mega

Plain Text

Display Active

Current Weather Displayed

Button Navigation Functional

ESP32-C3

Plain Text

Outdoor Uptime Increasing

Wind Updates Present

Rain Updates Present

ESP32-S3

Plain Text

Dashboard Updating

Database Uploads Active

Weather Underground ONLINE

E.4 Recovery Procedures

Communications Failure Recovery

If outdoor communications stop:

- Observe Ext NO condition.

- Verify outdoor uptime.

- Allow automatic recovery.

- Observe STATUS requests.

- Observe REBOOT attempts.

- Verify uptime reset after recovery.

Dashboard Recovery

If dashboard updates stop:

- Verify Arduino Mega operation.

- Verify UART communications.

- Verify ESP32-S3 operation.

- Verify browser connectivity.

- Restart ESP32-S3 if necessary.

Internet Recovery

If uploads fail:

- Verify WiFi connectivity.

- Verify local Internet access.

- Verify Weather Underground status.

- Verify database connectivity.

- Observe automatic recovery.

E.5 Seasonal Inspection Recommendations

Spring

Perform:

- Rain-gauge cleaning

- Wiring inspection

- Sensor inspection

Particular attention should be given to winter weather damage.

Summer

Perform:

- Solar-panel cleaning

- Radiation-shield inspection

- Temperature verification

Excessive contamination can affect summer temperature measurements.

Fall

Perform:

- Rain-gauge cleaning

- Leaf removal

- Sensor inspection

This is particularly important for maintaining rainfall accuracy.

Winter

Inspect:

- Battery health

- Solar charging performance

- Enclosure sealing

Cold-weather operation should be verified before severe weather arrives.

E.6 Documentation Management

The following records should be preserved throughout the service life of the weather station:

Technical Documents

Plain Text

Technical Manual

User Manual

Protocol Specifications

Wiring Diagrams

Firmware Archives

Plain Text

Arduino Mega Firmware

ESP32-S3 Firmware

ESP32-C3 Firmware

Every released version should be archived.

Installation Records

Maintain:

Plain Text

Installation Photos

Wiring Photos

Enclosure Photos

Sensor Alignment Records

These records greatly simplify future troubleshooting.

E.7 Backup Strategy

Firmware Backups

Backup before:

- Feature additions

- Pin changes

- Protocol modifications

- Sensor additions

Recommended archive structure:

Plain Text

Firmware

├── Mega

├── ESP32-S3

└── ESP32-C3

Database Backups

Recommended schedule:

Plain Text

Weekly

Minimum schedule:

Plain Text

Monthly

Historical weather data becomes increasingly valuable as the archive grows.

Documentation Backups

Backup after every major revision.

Recommended archive:

Plain Text

Technical Manual

User Manual

Installation Notes

Photos

Schematics

E.8 Spare Equipment Strategy

Recommended storage inventory:

Plain Text

1 Arduino Mega

1 ESP32-S3

1 ESP32-C3

1 LoRa Module

10 RJ Connectors

2 Sensor Cables

1 Battery

Miscellaneous Resistors

The objective is rapid recovery from common failures.

E.9 Long-Term Support Philosophy

The weather station was designed around three priorities:

Plain Text

Reliability

Serviceability

Expandability

Future modifications should preserve these goals whenever possible.

New features should not compromise:

- Existing weather acquisition

- Existing communications

- Existing recovery mechanisms

E.10 Release Acceptance Checklist

Before publication of a new system release:

Hardware

- Sensor wiring verified

- GPIO assignments verified

- Solar system verified

- Communications verified

Firmware

- Arduino firmware verified

- ESP32-S3 firmware verified

- ESP32-C3 firmware verified

Dashboard

- Data updates verified

- Status indicators verified

- Mobile layout verified

Uploads

- Database uploads verified

- Weather Underground uploads verified

Documentation

- Technical manual updated

- BOM updated

- Protocol documentation updated

- Revision history updated

E.11 Release Statement

Wireless Weather Station Technical Manual

Document ID: WWS-TM-001

Release: 1.0

This document defines the architecture, implementation, operation, maintenance, troubleshooting, and future development path of the Wireless Weather Station platform.

The manual shall serve as the authoritative engineering reference for future maintenance, enhancement, support, and replication of the system.

End of Volume E