Volume G

Engineering Notes and Design Rationale

The purpose of this volume is to preserve the engineering reasoning behind major design decisions. Future maintenance is significantly easier when system behavior can be traced back to the original design objectives.

G.1 Why a Three-Controller Architecture Was Selected

The weather station could have been implemented using a single microcontroller.

That approach was ultimately rejected because it would have created a controller responsible for:

- Weather measurements

- User interface

- Database communications

- Dashboard hosting

- LoRa communications

- Internet communications

By distributing responsibilities across three processors, each subsystem remains relatively independent.

Arduino Mega

Responsible for:

Plain Text

Weather Logic

LCD Interface

RTC

EEPROM

Recovery Management

ESP32-C3

Responsible for:

Plain Text

Outdoor Data Acquisition

LoRa Communications

Rain Processing

Wind Processing

ESP32-S3

Responsible for:

Plain Text

Dashboard

WiFi

Database Uploads

Weather Underground Uploads

This separation significantly simplifies maintenance and troubleshooting.

G.2 Why LoRa Was Selected

Several wireless technologies were considered.

WiFi

Advantages:

Plain Text

High Throughput

Easy Network Integration

Disadvantages:

Plain Text

Higher Power Consumption

Outdoor Coverage Challenges

Bluetooth

Advantages:

Plain Text

Low Cost

Simple Hardware

Disadvantages:

Plain Text

Limited Range

LoRa

Advantages:

Plain Text

Excellent Range

Low Power Consumption

Reliable Outdoor Operation

Simple Deployment

LoRa was selected because it provided the best combination of range and reliability for the outdoor sensor node.

G.3 Why Solar Power Was Adopted

The original concept assumed permanent wired power.

During development it became apparent that outdoor sensor placement and wiring flexibility would improve significantly if the outdoor node became self-powered.

Benefits achieved:

Plain Text

No power cable

Flexible installation

Reduced trenching

Improved placement options

G.4 Why Outdoor Uptime Was Added

Outdoor uptime was not part of the original design.

Troubleshooting revealed that determining whether the outdoor node had rebooted was often difficult.

The uptime counter was introduced to provide visibility into:

Plain Text

Unexpected Resets

Power Problems

Recovery Success

Communications Health

It became one of the most valuable diagnostic metrics in the entire system.

G.5 Why Remote Reboot Was Added

Field failures occasionally required access to the outdoor node.

Because the node could be mounted outdoors and operate on solar power, physical access is less convenient than access to indoor equipment.

The remote reboot protocol was created so that:

Plain Text

Communications Failures

Software Lockups

Temporary Faults

could often be recovered without a service visit.

G.6 Reliability Philosophy

The station was designed around a simple principle:

Plain Text

Collect Data First

Display Data Second

Upload Data Third

Weather acquisition should continue whenever possible.

Failures in:

Plain Text

Internet Connectivity

Database Services

Dashboard Services

should not prevent weather collection.

This principle strongly influenced the final architecture.

G.7 Design Principles

The weather station follows the principles below.

Simplicity

Prefer understandable solutions over sophisticated solutions.

Recoverability

Subsystems should recover automatically whenever practical.

Visibility

Failures should be obvious.

Documentation

Critical behaviors should be documented.

Expandability

Future sensors should be easy to add.

G.8 Long-Term Support Objectives

The project was designed with an anticipated service life measured in years rather than months.

Long-term goals include:

Plain Text

Minimal Maintenance

Predictable Operation

Simple Repair

Incremental Enhancement

The technical manual exists partly to support these objectives.

End of Volume G